DATABLAD

2N® IP SAFETY

ROBUST IP-PORTTELEFON FÖR NÖDSITUATIONER

2N® IP Safety är en förstklassig säkerhetsporttelefon som är utformad för nödkommunikation. Oavsett om den dränks av regn eller utsätts för

-40°C frost förblir den fullt funktionsduglig och låter dig nå hjälp på en gång och dess ljusa orange färg gör den lätt att hitta i dålig sikt.

- Robust metallhölje

- Snabb hjälp vid nödsituationer

- Hög uppfattbarhet

- Automatisk test av ljusslinga

- Integration i ditt befintliga system

VARIANTER

2 knappar - Info och SOS, 10W högtalare

9152102W

1 knapp, 10W högtalare

9152101W

Röd nödknapp, 10W

högtalare

9152101MW

TECHNICAL PARAMETERS

| Signalling protocol |  | Mechanical properties |  |
| --- | --- | --- | --- |
| SIP 2.0 (RFC - 3261) |  | Cover | Robust aluminium cast product |
|  |  | Operating temperature | −40 °C to 55 °C |
| Button |  | Storage temperature | −40 °C to 70 °C |
| Button design | Metal buttons with blue backlight or a red emergency button | Operating relative humidity |  |
|  |  |  | 10 % – 95 % (non-condensing) |
| Number of buttons | 1 or 2 | Dimensions | 217 x 109 x 83 mm |
| Numeric keypad | No |  | 242 x 136 x 83 mm incl. frame |
|  |  | Weight | netto max. 2 kg / brutto max. 2,5 kg |
| Audio |  | Coverage level | IP69K and IK10 |
| Microphone | 2 integrated microphones |  |  |
| Amplifier | 10 W (class D) |  |  |
| Loudspeaker | 10 W |  |  |
| Sound pressure level (SPL max) | 94 dB (10 W type, for 1 kHz, distance 1 m) |  |  |
| Volume control | Adjustable with automatic adaptive mode |  |  |
| Full duplex | Yes (AEC) |  |  |
| Audio stream |  |  |  |
| Protocols | RTP / RTSP |  |  |
| Codecs | G.711, G.729, G.722, L16/16 kHz |  |  |
| Interfaces |  |  |  |
| Power supply | PoE or 12V / 2A DC |  |  |
| PoE | 802.3af (Class 0 - 12.95W) |  |  |
| LAN | 10/100BASE-TX s Auto-MDIX, RJ-45 |  |  |
| Recommended cabling | Cat-5e or higher |  |  |
| Supported protocols | SIP2.0, DHCP opt. 66, SMTP, 802.1x, RTSP, RTP, TFTP, HTTP, HTTPS, Syslog, ONVIF |  |  |
| Passive switch | NO/NC contacts, up to 30 V / 1 A AC/DC |  |  |
| Active switch output | 9 up to 13 V DC depending on power supply (PoE: 9 V; adaptor: power supply voltage minus 1 V), max 600 mA |  |  |

2N TELEKOMUNIKACE a.s., Modřanská 621, 143 01 Prague 4, CZ, +420 261 301 500, sales@2n.cz, www.2n.cz